Wydział Informatyki Politechniki

Białostockiej Przetwarzanie Sygnałów

i Obrazów – Pracownia specjalistyczna

Ćwiczenie Nr. 4

Temat: Analiza widmowa sygnałów.

Imię i nazwisko studenta: Michał Pączkowski.

Data realizacji ćwiczenia : 20.04.2026

Wszystkie zdjęcia w: SprawozdaniePsIO4 na github.

Link do repozytorium: https://github.com/ErythrinaCremesinus/PsIO_Sprawko4

Zadanie 4.1

Interpretacja:

- szum gaussowski: obwiednia powinna być w przybliżeniu stała, bez wyraźnego trendu

- sinusoida 1 kHz: obwiednia szybko zbiega do stałego poziomu

- chirp: obwiednia zmienia się w czasie, bo własności sygnału zależą od chwili")

- sygnał mowy: obwiednia ma wyraźne odcinki o różnej energii, więc sygnał jest niestacjonarny")

Kod:

from __future__ import annotations

from pathlib import Path

import numpy as np

import matplotlib.pyplot as plt

FS = 8000

DT = 1.0 / FS

def recursive_power_envelope(signal: np.ndarray, alpha: float) -> np.ndarray:

power = np.empty_like(signal, dtype=float)

power[0] = signal[0] ** 2

for index in range(1, len(signal)):

power[index] = alpha * power[index - 1] + (1.0 - alpha) * signal[index] ** 2

return power

def time_axis(duration: float) -> np.ndarray:

return np.arange(int(duration * FS)) / FS

def generate_gaussian_noise(duration: float = 3.0) -> tuple[np.ndarray, np.ndarray]:

rng = np.random.default_rng(42)

t = time_axis(duration)

signal = rng.normal(0.0, 1.0, size=t.shape)

return t, signal

def generate_sine(duration: float = 3.0, frequency: float = 1000.0) -> tuple[np.ndarray, np.ndarray]:

t = time_axis(duration)

signal = np.sin(2.0 * np.pi * frequency * t)

return t, signal

def generate_chirp(duration: float = 5.0, f0: float = 0.0, f1: float = 1000.0) -> tuple[np.ndarray, np.ndarray]:

t = time_axis(duration)

k = (f1 - f0) / duration

phase = 2.0 * np.pi * (f0 * t + 0.5 * k * t**2)

signal = np.sin(phase)

return t, signal

def generate_speech_like(duration: float = 3.0) -> tuple[np.ndarray, np.ndarray]:

t = time_axis(duration)

signal = np.zeros_like(t)

segments = [

(0.0, 0.8, 120.0, 0.20, 700.0),

(0.8, 1.6, 140.0, 0.30, 900.0),

(1.6, 2.1, None, 0.12, None),

(2.1, 3.0, 110.0, 0.25, 500.0),

]

for start, end, pitch, amplitude, formant in segments:

mask = (t >= start) & (t < end)

local_t = t[mask] - start

if pitch is None:

rng = np.random.default_rng(7)

noise = rng.normal(0.0, 1.0, size=local_t.shape)

envelope = 0.7 + 0.3 * np.sin(2.0 * np.pi * 3.0 * local_t) ** 2

signal[mask] = amplitude * envelope * noise

continue

glottal = (

np.sin(2.0 * np.pi * pitch * local_t)

+ 0.5 * np.sin(2.0 * np.pi * 2.0 * pitch * local_t)

+ 0.25 * np.sin(2.0 * np.pi * 3.0 * pitch * local_t)

)

modulator = 0.6 + 0.4 * np.sin(2.0 * np.pi * 1.5 * local_t) ** 2

if formant is not None:

resonance = 1.0 + 0.4 * np.sin(2.0 * np.pi * formant * local_t / FS)

else:

resonance = 1.0

signal[mask] = amplitude * modulator * resonance * glottal

return t, signal

def load_or_generate_speech() -> tuple[np.ndarray, np.ndarray]:

script_dir = Path(__file__).resolve().parent

candidates = sorted(script_dir.glob("*.wav"))

if candidates:

try:

from scipy.io import wavfile

sample_rate, data = wavfile.read(candidates[0])

if data.ndim > 1:

data = data[:, 0]

data = data.astype(float)

if np.max(np.abs(data)) > 0:

data /= np.max(np.abs(data))

if sample_rate != FS:

duration = len(data) / sample_rate

old_t = np.linspace(0.0, duration, len(data), endpoint=False)

new_t = time_axis(duration)

data = np.interp(new_t, old_t, data)

return new_t, data

return time_axis(len(data) / sample_rate), data

except Exception:

pass

return generate_speech_like()

def plot_signal_with_envelopes(ax, t: np.ndarray, signal: np.ndarray, title: str, alphas: list[float]) -> None:

ax.plot(t, signal, color="0.7", linewidth=0.8, label="sygnał")

for alpha in alphas:

envelope = recursive_power_envelope(signal, alpha)

ax.plot(t, envelope, linewidth=1.4, label=f"P[n], α={alpha}")

ax.set_title(title)

ax.set_xlabel("czas [s]")

ax.set_ylabel("amplituda / moc")

ax.grid(True, alpha=0.25)

def main() -> None:

alpha_values = [0.9, 0.99, 0.999]

signals = [

("Szum gaussowski", *generate_gaussian_noise(3.0)),

("Sinusoida 1 kHz", *generate_sine(3.0, 1000.0)),

("Chirp 0 Hz -> 1 kHz", *generate_chirp(5.0, 0.0, 1000.0)),

("Sygnał mowy", *load_or_generate_speech()),

]

fig, axes = plt.subplots(2, 2, figsize=(15, 9), constrained_layout=True)

axes = axes.ravel()

for ax, (title, t, signal) in zip(axes, signals):

plot_signal_with_envelopes(ax, t, signal, title, alpha_values)

ax.legend(loc="upper right", fontsize=9)

fig.suptitle("Zadanie 4.1 - obwiednia mocy liczona rekurencyjnie", fontsize=16)

backend = plt.get_backend().lower()

if "agg" in backend or "inline" in backend:

output_path = Path(__file__).with_name("zad4_1_wykresy.png")

fig.savefig(output_path, dpi=200)

print(f"Zapisano wykres do: {output_path}")

else:

plt.show()

if __name__ == "__main__":

main()

Zadanie 4.2

Kod:

from __future__ import annotations

from pathlib import Path

import matplotlib.pyplot as plt

import numpy as np

FS = 8000

N = 1024

F1 = 1000.0

F2 = 2000.0

def generate_signals() -> dict[str, np.ndarray]:

rng = np.random.default_rng(12345)

n = np.arange(N)

w = rng.normal(0.0, 1.0, size=N)

s = 0.5 * np.sin(2.0 * np.pi * F1 * n / FS) + np.sin(2.0 * np.pi * F2 * n / FS)

y = s + 0.1 * w

return {"w[n]": w, "s[n]": s, "y[n]": y}

def safe_db(values: np.ndarray, reference: float = 1.0) -> np.ndarray:

return 10.0 * np.log10(np.maximum(values, 1e-15) / reference)

def one_sided_frequency_axis(length: int, fs: float = FS) -> np.ndarray:

return np.fft.rfftfreq(length, d=1.0 / fs)

def periodogram(signal: np.ndarray, fs: float = FS) -> tuple[np.ndarray, np.ndarray]:

length = len(signal)

spectrum = np.fft.rfft(signal)

power = (np.abs(spectrum) ** 2) / length

if power.size > 2:

power[1:-1] *= 2.0

return one_sided_frequency_axis(length, fs), power

def fft_power_spectrum(signal: np.ndarray, fs: float = FS, nfft: int | None = None) -> tuple[np.ndarray, np.ndarray]:

if nfft is None:

nfft = len(signal)

spectrum = np.fft.rfft(signal, n=nfft)

power = (np.abs(spectrum) ** 2) / len(signal)

if power.size > 2:

power[1:-1] *= 2.0

return one_sided_frequency_axis(nfft, fs), power

def hann_window_spectrum(signal: np.ndarray, window_length: int, fs: float = FS) -> tuple[np.ndarray, np.ndarray]:

window = np.hanning(window_length)

windowed = signal[:window_length] * window

coherent_gain = np.sum(window) / window_length

if coherent_gain == 0:

coherent_gain = 1.0

spectrum = np.fft.rfft(windowed, n=window_length)

power = (np.abs(spectrum) ** 2) / (window_length * coherent_gain**2)

if power.size > 2:

power[1:-1] *= 2.0

return one_sided_frequency_axis(window_length, fs), power

def plot_spectrum(ax, freqs: np.ndarray, power: np.ndarray, label: str, style: str = "-", linewidth: float = 1.2) -> None:

ax.plot(freqs, safe_db(power), style, linewidth=linewidth, label=label)

ax.set_xlim(0, FS / 2)

ax.set_xlabel("częstotliwość [Hz]")

ax.set_ylabel("poziom [dB]")

ax.grid(True, alpha=0.25)

def analyze_signal(signal_name: str, signal: np.ndarray) -> tuple[plt.Figure, plt.Figure]:

fig, axes = plt.subplots(2, 2, figsize=(15, 9), constrained_layout=True)

axes = axes.ravel()

freqs_p, power_p = periodogram(signal)

plot_spectrum(axes[0], freqs_p, power_p, "periodogram")

axes[0].set_title(f"{signal_name} - periodogram")

axes[0].legend(loc="upper right")

freqs_f, power_f = fft_power_spectrum(signal)

plot_spectrum(axes[1], freqs_f, power_f, "FFT", linewidth=1.0)

axes[1].set_title(f"{signal_name} - FFT")

axes[1].legend(loc="upper right")

for ax, window_length in zip(axes[2:], [256, 128]):

freqs_w, power_w = hann_window_spectrum(signal, window_length)

plot_spectrum(ax, freqs_w, power_w, f"Hann {window_length}")

ax.set_title(f"{signal_name} - FFT z oknem Hanna {window_length}")

ax.legend(loc="upper right")

extra_fig, extra_ax = plt.subplots(figsize=(8, 4.8), constrained_layout=True)

freqs_w64, power_w64 = hann_window_spectrum(signal, 64)

plot_spectrum(extra_ax, freqs_w64, power_w64, "Hann 64")

extra_ax.set_title(f"{signal_name} - FFT z oknem Hanna 64")

extra_ax.legend(loc="upper right")

fig.suptitle(f"Zadanie 4.2 - analiza widmowa: {signal_name}", fontsize=16)

extra_fig.suptitle(f"Zadanie 4.2 - analiza widmowa: {signal_name} (okno 64)", fontsize=16)

return fig, extra_fig

def save_or_show(figures: list[tuple[str, plt.Figure]]) -> None:

backend = plt.get_backend().lower()

if "agg" in backend or "inline" in backend:

output_dir = Path(__file__).resolve().parent

for name, fig in figures:

output_path = output_dir / f"zad4_2_{name}.png"

fig.savefig(output_path, dpi=200)

print(f"Zapisano wykres: {output_path}")

else:

plt.show()

def main() -> None:

signals = generate_signals()

figures: list[tuple[str, plt.Figure]] = []

for name, signal in signals.items():

fig_main, fig_64 = analyze_signal(name, signal)

safe_name = name.replace("[", "").replace("]", "").replace(" ", "_")

figures.append((f"{safe_name}_main", fig_main))

figures.append((f"{safe_name}_hann64", fig_64))

save_or_show(figures)

if __name__ == "__main__":

main()

Wnioski:

- w[n] ma widmo szerokopasmowe, bez wyraźnych linii widmowych

- s[n] ma dwa dominujące piki przy 1 kHz i 2 kHz

- y[n] zachowuje piki s[n], ale ma podniesione tło szumowe

- Okno Hanna zmniejsza przecieki widmowe; krótsze okna dają gorszą rozdzielczość częstotliwościową

Zadanie 4.3

Kod:

from __future__ import annotations

import argparse

from pathlib import Path

import matplotlib.pyplot as plt

import numpy as np

try:

from scipy.io import wavfile

from scipy.signal import spectrogram

except Exception as exc:  # pragma: no cover - depends on environment

raise RuntimeError(

"Do uruchomienia tego skryptu potrzebne są pakiety scipy i matplotlib."

) from exc

WINDOW_LENGTH = 1024

OVERLAP = 768

DEFAULT_FALLBACK_FS = 8000

def parse_args() -> argparse.Namespace:

parser = argparse.ArgumentParser(description="Zadanie 4.3 - analiza sygnału")

parser.add_argument(

"input",

nargs="?",

default=None,

help="Ścieżka do pliku WAV. Jeśli brak, skrypt użyje pierwszego WAV z katalogu lub sygnału demonstracyjnego.",

)

return parser.parse_args()

def normalize_signal(signal: np.ndarray) -> np.ndarray:

signal = signal.astype(float)

peak = np.max(np.abs(signal))

if peak > 0:

signal = signal / peak

return signal

def generate_demo_signal(fs: int = DEFAULT_FALLBACK_FS, duration: float = 3.0) -> tuple[int, np.ndarray]:

t = np.arange(int(fs * duration)) / fs

signal = np.zeros_like(t)

first = t < 1.0

second = (t >= 1.0) & (t < 2.0)

third = t >= 2.0

signal[first] = 0.9 * np.sin(2.0 * np.pi * 200.0 * t[first])

signal[second] = 0.6 * np.sin(2.0 * np.pi * (400.0 * t[second] + 200.0 * t[second] ** 2))

rng = np.random.default_rng(7)

signal[third] = 0.15 * rng.normal(0.0, 1.0, size=np.count_nonzero(third))

return fs, signal

def find_default_wav(script_dir: Path) -> Path | None:

preferred = script_dir / "4.3.wav"

if preferred.exists():

return preferred

candidates = sorted(script_dir.glob("*.wav"))

return candidates[0] if candidates else None

def load_signal(input_path: str | None, script_dir: Path) -> tuple[int, np.ndarray, str]:

if input_path:

path = Path(input_path)

else:

path = find_default_wav(script_dir)

if path is None:

fs, signal = generate_demo_signal()

return fs, signal, "sygnał demonstracyjny"

fs, data = wavfile.read(path)

if data.ndim > 1:

data = np.mean(data, axis=1)

signal = normalize_signal(data)

return int(fs), signal, str(path)

def safe_db(values: np.ndarray, reference: float = 1.0) -> np.ndarray:

return 10.0 * np.log10(np.maximum(values, 1e-15) / reference)

def one_sided_fft(signal: np.ndarray, fs: int) -> tuple[np.ndarray, np.ndarray]:

spectrum = np.fft.rfft(signal)

freqs = np.fft.rfftfreq(len(signal), d=1.0 / fs)

amplitude = np.abs(spectrum) / len(signal)

if amplitude.size > 2:

amplitude[1:-1] *= 2.0

return freqs, amplitude

def compute_spectrogram(signal: np.ndarray, fs: int) -> tuple[np.ndarray, np.ndarray, np.ndarray]:

freqs, times, sxx = spectrogram(

signal,

fs=fs,

window="hamming",

nperseg=WINDOW_LENGTH,

noverlap=OVERLAP,

nfft=WINDOW_LENGTH,

scaling="density",

mode="magnitude",

)

power = sxx**2

return freqs, times, power

def plot_time_domain(ax, signal: np.ndarray, fs: int, title: str) -> None:

time = np.arange(len(signal)) / fs

ax.plot(time, signal, color="#1f77b4", linewidth=1.0)

ax.set_title(title)

ax.set_xlabel("czas [s]")

ax.set_ylabel("amplituda")

ax.grid(True, alpha=0.25)

def plot_fft(ax, signal: np.ndarray, fs: int, title: str) -> None:

freqs, amplitude = one_sided_fft(signal, fs)

ax.plot(freqs, safe_db(amplitude), color="#d62728", linewidth=1.0)

ax.set_title(title)

ax.set_xlabel("częstotliwość [Hz]")

ax.set_ylabel("amplituda [dB]")

ax.grid(True, alpha=0.25)

ax.set_xlim(0, fs / 2)

def plot_spectrogram(ax, signal: np.ndarray, fs: int, title: str) -> None:

freqs, times, power = compute_spectrogram(signal, fs)

pcm = ax.pcolormesh(times, freqs, safe_db(power), shading="auto", cmap="magma")

ax.set_title(title)

ax.set_xlabel("czas [s]")

ax.set_ylabel("częstotliwość [Hz]")

ax.set_ylim(0, fs / 2)

return pcm

def save_or_show(figures: list[tuple[str, plt.Figure]]) -> None:

for name, fig in figures:

output = Path(__file__).with_name(f"zad4_3_{name}.png")

fig.savefig(output, dpi=200)

print(f"Zapisano wykres: {output}")

backend = plt.get_backend().lower()

if "agg" not in backend and "inline" not in backend:

plt.show()

def create_analysis_figure(signal: np.ndarray, fs: int, source_name: str) -> plt.Figure:

fig, axes = plt.subplots(3, 1, figsize=(15, 11), constrained_layout=True)

plot_time_domain(axes[0], signal, fs, f"{source_name} - dziedzina czasu")

plot_fft(axes[1], signal, fs, f"{source_name} - FFT")

pcm = plot_spectrogram(axes[2], signal, fs, f"{source_name} - spektrogram Hamming {WINDOW_LENGTH}")

fig.colorbar(pcm, ax=axes[2], label="poziom mocy [dB]")

fig.suptitle("Zadanie 4.3 - analiza sygnału w czasie i częstotliwości", fontsize=16)

return fig

def main() -> None:

args = parse_args()

script_dir = Path(__file__).resolve().parent

fs, signal, source_name = load_signal(args.input, script_dir)

fig = create_analysis_figure(signal, fs, source_name)

save_or_show([(Path(source_name).stem.replace(".", "_").replace(" ", "_"), fig)])

duration = len(signal) / fs

if __name__ == "__main__":

main()

Wnioski:

- Sygnał ma długość {duration:.3f} s i jest próbkowany z fs = {fs} Hz

- FFT pokazuje globalny rozkład energii w całym sygnale

- Spektrogram pokazuje, jak energia zmienia się w czasie i częstotliwości

- Dla modułu STFT i widma mocy w dB obowiązuje zależność 10log10(|X|^2) = 20log10(|X|)

- Jeśli sygnał jest mowy lub modulowany, energia zwykle skupia się w pasmach formantowych i w krótkich odcinkach czasu

Zadanie 4.4

Kod:

from __future__ import annotations

from pathlib import Path

import matplotlib.pyplot as plt

import numpy as np

try:

from scipy.io import wavfile

from scipy.signal import spectrogram as scipy_spectrogram

except Exception as exc:  # pragma: no cover - environment dependent

raise RuntimeError("Do uruchomienia wymagane są pakiety scipy i matplotlib.") from exc

WINDOW_LENGTH = 1024

OVERLAP = 768

HOP = WINDOW_LENGTH - OVERLAP

DEFAULT_FALLBACK_FS = 8000

def normalize_signal(signal: np.ndarray) -> np.ndarray:

signal = signal.astype(float)

peak = np.max(np.abs(signal))

if peak > 0:

signal = signal / peak

return signal

def generate_demo_signal(fs: int = DEFAULT_FALLBACK_FS, duration: float = 3.0) -> tuple[int, np.ndarray]:

t = np.arange(int(fs * duration)) / fs

signal = np.zeros_like(t)

first = t < 1.0

second = (t >= 1.0) & (t < 2.0)

third = t >= 2.0

signal[first] = 0.9 * np.sin(2.0 * np.pi * 220.0 * t[first])

signal[second] = 0.6 * np.sin(2.0 * np.pi * (300.0 * t[second] + 250.0 * t[second] ** 2))

rng = np.random.default_rng(11)

signal[third] = 0.12 * rng.normal(0.0, 1.0, size=np.count_nonzero(third))

return fs, signal

def find_default_wav(script_dir: Path) -> Path | None:

candidates = sorted(script_dir.glob("*.wav"))

return candidates[0] if candidates else None

def load_signal(script_dir: Path) -> tuple[int, np.ndarray, str]:

path = find_default_wav(script_dir)

if path is None:

fs, signal = generate_demo_signal()

return fs, signal, "sygnał demonstracyjny"

fs, data = wavfile.read(path)

if data.ndim > 1:

data = np.mean(data, axis=1)

signal = normalize_signal(data)

return int(fs), signal, str(path)

def safe_db(values: np.ndarray) -> np.ndarray:

return 10.0 * np.log10(np.maximum(values, 1e-15))

def one_sided_fft(signal: np.ndarray, fs: int) -> tuple[np.ndarray, np.ndarray]:

spectrum = np.fft.rfft(signal)

freqs = np.fft.rfftfreq(len(signal), d=1.0 / fs)

amplitude = np.abs(spectrum) / len(signal)

if amplitude.size > 2:

amplitude[1:-1] *= 2.0

return freqs, amplitude

def custom_smoothed_periodogram_spectrogram(

signal: np.ndarray,

fs: int,

window_length: int = WINDOW_LENGTH,

overlap: int = OVERLAP,

time_smoothing: int = 3,

) -> tuple[np.ndarray, np.ndarray, np.ndarray]:

hop = window_length - overlap

if hop <= 0:

raise ValueError("overlap musi być mniejszy od window_length")

window = np.hamming(window_length)

window_energy = np.sum(window**2)

if window_energy == 0:

window_energy = 1.0

if len(signal) < window_length:

signal = np.pad(signal, (0, window_length - len(signal)))

starts = np.arange(0, len(signal) - window_length + 1, hop)

if starts.size == 0:

starts = np.array([0])

spectrum_columns = []

times = []

for start in starts:

frame = signal[start : start + window_length]

if len(frame) < window_length:

frame = np.pad(frame, (0, window_length - len(frame)))

windowed = frame * window

spectrum = np.fft.rfft(windowed, n=window_length)

periodogram = (np.abs(spectrum) ** 2) / (fs * window_energy)

if periodogram.size > 2:

periodogram[1:-1] *= 2.0

spectrum_columns.append(periodogram)

times.append((start + window_length / 2.0) / fs)

power = np.stack(spectrum_columns, axis=1)

if time_smoothing > 1:

kernel = np.ones(time_smoothing) / time_smoothing

smoothed = np.empty_like(power)

for bin_index in range(power.shape[0]):

smoothed[bin_index, :] = np.convolve(power[bin_index, :], kernel, mode="same")

power = smoothed

freqs = np.fft.rfftfreq(window_length, d=1.0 / fs)

return freqs, np.asarray(times), power

def reference_spectrogram(signal: np.ndarray, fs: int) -> tuple[np.ndarray, np.ndarray, np.ndarray]:

freqs, times, sxx = scipy_spectrogram(

signal,

fs=fs,

window="hamming",

nperseg=WINDOW_LENGTH,

noverlap=OVERLAP,

nfft=WINDOW_LENGTH,

scaling="density",

mode="psd",

)

return freqs, times, sxx

def plot_time_domain(ax, signal: np.ndarray, fs: int, title: str) -> None:

time = np.arange(len(signal)) / fs

ax.plot(time, signal, color="#1f77b4", linewidth=0.9)

ax.set_title(title)

ax.set_xlabel("czas [s]")

ax.set_ylabel("amplituda")

ax.grid(True, alpha=0.25)

def plot_fft(ax, signal: np.ndarray, fs: int, title: str) -> None:

freqs, amplitude = one_sided_fft(signal, fs)

ax.plot(freqs, safe_db(amplitude), color="#d62728", linewidth=0.9)

ax.set_title(title)

ax.set_xlabel("częstotliwość [Hz]")

ax.set_ylabel("amplituda [dB]")

ax.set_xlim(0, fs / 2)

ax.grid(True, alpha=0.25)

def plot_3d_mesh(ax, freqs: np.ndarray, times: np.ndarray, power: np.ndarray, title: str) -> None:

time_grid, freq_grid = np.meshgrid(times, freqs)

surface = ax.plot_surface(

time_grid,

freq_grid,

safe_db(power),

cmap="magma",

linewidth=0,

antialiased=True,

rcount=min(80, power.shape[0]),

ccount=min(80, power.shape[1]),

)

ax.set_title(title)

ax.set_xlabel("czas [s]")

ax.set_ylabel("częstotliwość [Hz]")

ax.set_zlabel("gęstość mocy [dB]")

ax.set_ylim(0, freqs.max())

return surface

def save_figures(figures: list[tuple[str, plt.Figure]]) -> None:

for name, fig in figures:

output = Path(__file__).with_name(f"zad4_4_{name}.png")

fig.savefig(output, dpi=200)

print(f"Zapisano wykres: {output}")

def create_main_figure(signal: np.ndarray, fs: int, source_name: str) -> plt.Figure:

fig = plt.figure(figsize=(16, 8), constrained_layout=True)

grid = fig.add_gridspec(2, 1)

ax_time = fig.add_subplot(grid[0, 0])

ax_fft = fig.add_subplot(grid[1, 0])

plot_time_domain(ax_time, signal, fs, f"{source_name} - dziedzina czasu")

plot_fft(ax_fft, signal, fs, f"{source_name} - FFT")

fig.suptitle("Zadanie 4.4 - sygnał i FFT", fontsize=16)

return fig

def create_3d_figure(signal: np.ndarray, fs: int, source_name: str) -> plt.Figure:

fig = plt.figure(figsize=(16, 9), constrained_layout=True)

ax_mesh = fig.add_subplot(111, projection="3d")

freqs, times, power = custom_smoothed_periodogram_spectrogram(signal, fs)

surface = plot_3d_mesh(ax_mesh, freqs, times, power, f"{source_name} - własny spektrogram 3D")

fig.colorbar(surface, ax=ax_mesh, shrink=0.8, pad=0.1, label="gęstość mocy [dB]")

fig.suptitle("Zadanie 4.4 - własny spektrogram 3D", fontsize=16)

return fig

def create_comparison_figure(signal: np.ndarray, fs: int, source_name: str) -> plt.Figure:

custom_freqs, custom_times, custom_power = custom_smoothed_periodogram_spectrogram(signal, fs)

ref_freqs, ref_times, ref_power = reference_spectrogram(signal, fs)

fig, axes = plt.subplots(1, 2, figsize=(16, 6), constrained_layout=True)

mesh0 = axes[0].pcolormesh(custom_times, custom_freqs, safe_db(custom_power), shading="auto", cmap="magma")

axes[0].set_title("Własny spektrogram")

axes[0].set_xlabel("czas [s]")

axes[0].set_ylabel("częstotliwość [Hz]")

axes[0].set_ylim(0, fs / 2)

fig.colorbar(mesh0, ax=axes[0], label="gęstość mocy [dB]")

mesh1 = axes[1].pcolormesh(ref_times, ref_freqs, safe_db(ref_power), shading="auto", cmap="magma")

axes[1].set_title("Referencyjny spektrogram SciPy")

axes[1].set_xlabel("czas [s]")

axes[1].set_ylabel("częstotliwość [Hz]")

axes[1].set_ylim(0, fs / 2)

fig.colorbar(mesh1, ax=axes[1], label="gęstość mocy [dB]")

fig.suptitle(f"Zadanie 4.4 - porównanie spektrogramów: {source_name}", fontsize=16)

return fig

def main() -> None:

script_dir = Path(__file__).resolve().parent

fs, signal, source_name = load_signal(script_dir)

main_fig = create_main_figure(signal, fs, source_name)

three_d_fig = create_3d_figure(signal, fs, source_name)

comparison_fig = create_comparison_figure(signal, fs, source_name)

safe_name = Path(source_name).stem.replace(".", "_").replace(" ", "_")

save_figures([

(f"{safe_name}_main", main_fig),

(f"{safe_name}_3d", three_d_fig),

(f"{safe_name}_compare", comparison_fig),

])

plt.show()

duration = len(signal) / fs

if __name__ == "__main__":

main()

Wnioski:

- Sygnał ma długość {duration:.3f} s i jest próbkowany z fs = {fs} Hz

- FFT pokazuje globalny rozkład energii, a spektrogram pokazuje jego zmianę w czasie

- Własny spektrogram oparty na periodogramach powinien być bardzo zbliżony do referencji SciPy

- Wygładzanie po czasie zmniejsza ziarnistość obrazu kosztem rozdzielczości czasowej

- Wynik w skali dB dobrze uwidacznia lokalne maksima energii i tło szumowe

Zadanie 4.5

Kod:

from __future__ import annotations

from pathlib import Path

import matplotlib.pyplot as plt

import numpy as np

try:

from noise import pnoise2

_USE_LIBRARY = True

except ModuleNotFoundError:

_USE_LIBRARY = False

def _fade(t: np.ndarray) -> np.ndarray:

return t * t * t * (t * (t * 6.0 - 15.0) + 10.0)

def _lerp(a: np.ndarray, b: np.ndarray, t: np.ndarray) -> np.ndarray:

return a + t * (b - a)

def _build_permutation(seed: int = 0) -> np.ndarray:

rng = np.random.default_rng(seed)

p = np.arange(256, dtype=int)

rng.shuffle(p)

return np.tile(p, 2)

def _gradient(h: np.ndarray, x: np.ndarray, y: np.ndarray) -> np.ndarray:

vectors = np.array([[1, 1], [-1, 1], [1, -1], [-1, -1],

[1, 0], [-1, 0], [0, 1], [0, -1]], dtype=float)

g = vectors[h % 8]

return g[..., 0] * x + g[..., 1] * y

def _perlin2(x: np.ndarray, y: np.ndarray, perm: np.ndarray) -> np.ndarray:

xi = np.floor(x).astype(int) & 255

yi = np.floor(y).astype(int) & 255

xf = x - np.floor(x)

yf = y - np.floor(y)

u = _fade(xf)

v = _fade(yf)

aa = perm[perm[xi    ] + yi    ]

ab = perm[perm[xi    ] + yi + 1]

ba = perm[perm[xi + 1] + yi    ]

bb = perm[perm[xi + 1] + yi + 1]

x1 = _lerp(_gradient(aa, xf,     yf    ),

_gradient(ba, xf - 1, yf    ), u)

x2 = _lerp(_gradient(ab, xf,     yf - 1),

_gradient(bb, xf - 1, yf - 1), u)

return _lerp(x1, x2, v)

def generate_noise_map_numpy(

width: int,

height: int,

scale: float = 4.0,

octaves: int = 6,

persistence: float = 0.5,

lacunarity: float = 2.0,

seed: int = 42,

) -> np.ndarray:

"""Generuje mapę szumu Perlin przez sumowanie oktaw (fBm)."""

perm = _build_permutation(seed)

xs = np.linspace(0, scale, width, endpoint=False)

ys = np.linspace(0, scale, height, endpoint=False)

xg, yg = np.meshgrid(xs, ys)

noise_map = np.zeros((height, width))

amplitude = 1.0

frequency = 1.0

max_val = 0.0

for _ in range(octaves):

noise_map += amplitude * _perlin2(xg * frequency, yg * frequency, perm)

max_val += amplitude

amplitude *= persistence

frequency *= lacunarity

return noise_map / max_val

def generate_noise_map_library(

width: int,

height: int,

scale: float = 4.0,

octaves: int = 6,

persistence: float = 0.5,

lacunarity: float = 2.0,

seed: int = 42,

) -> np.ndarray:

"""Generuje mapę szumu Perlin z użyciem pnoise2."""

noise_map = np.empty((height, width))

for row in range(height):

for col in range(width):

x = col / width * scale

y = row / height * scale

noise_map[row, col] = pnoise2(

x, y,

octaves=octaves,

persistence=persistence,

lacunarity=lacunarity,

base=seed,

)

return noise_map

def generate_noise_map(

width: int = 512,

height: int = 512,

scale: float = 4.0,

octaves: int = 6,

persistence: float = 0.5,

lacunarity: float = 2.0,

seed: int = 42,

) -> np.ndarray:

if _USE_LIBRARY:

print("Użyto: pnoise2 z biblioteki noise")

return generate_noise_map_library(width, height, scale, octaves, persistence, lacunarity, seed)

else:

print("Użyto: własna implementacja Perlin 2D (numpy)")

return generate_noise_map_numpy(width, height, scale, octaves, persistence, lacunarity, seed)

TERRAIN_LEVELS = [

(-1.00, -0.15, ( 2,  50, 127)),

(-0.15,  0.00, ( 0, 128, 205)),

( 0.00,  0.08, (231, 203, 153)),

( 0.08,  0.25, (101, 179,  22)),

( 0.25,  0.40, ( 2,  75,  26)),

( 0.40,  1.01, (203, 178,  78)),

]

def colorize(noise_map: np.ndarray) -> np.ndarray:

height, width = noise_map.shape

rgb = np.zeros((height, width, 3), dtype=np.uint8)

for lo, hi, color in TERRAIN_LEVELS:

mask = (noise_map >= lo) & (noise_map < hi)

for channel, value in enumerate(color):

rgb[..., channel][mask] = value

return rgb

def main() -> None:

WIDTH, HEIGHT = 512, 512

noise_map = generate_noise_map(

width=WIDTH,

height=HEIGHT,

scale=4.0,

octaves=6,

persistence=0.5,

lacunarity=2.0,

seed=42,

)

rgb = colorize(noise_map)

fig, ax = plt.subplots(figsize=(8, 8), constrained_layout=True)

ax.imshow(rgb, interpolation="nearest")

ax.axis("off")

fig.suptitle("Zadanie 4.5 - mapa terenu z szumu Perlin 2D", fontsize=14)

output = Path(__file__).with_name("zad4_5_mapa.png")

fig.savefig(output, dpi=200)

print(f"Zapisano: {output}")

plt.show()

if __name__ == "__main__":

main()

Wnioski:

Mimo podjęcia starań w celu wiernego odwzorowania mapy przy użyciu algorytmu szumu (noise), ostateczny rezultat wykazuje pewne odstępstwa od wzorca. Choć paleta barw oraz układ warstw zostały dobrane z dużą precyzją, trudność sprawiło zachowanie identycznej struktury i zagęszczenia biomów, w szczególności w obszarach leśnych oraz górskich (pustynnych?).

Oryginalna mapa:				   Mapa z kodu: